CLF353

- -   AGRICULTURAL CORE CURRICULUM  - -

(CLF300)  Core Area:  PLANT SCIENCE

(CLF350)  Unit Title:  IRRIGATION

____________________________________________________________________________

(CLF353)   Topic:  KNOWING WHEN             Time      Taught in Year(s)

TO IRRIGATE            2 hours            1

____________________________________________________________________________

Topic Objectives:  Upon completion of this lesson the student will

be able to:

Learning Outcome #:

(D-3)  -  Explain three ways of determining when to irrigate.

Special Materials and Equipment:

References: Janick, Schery, Woods, and Ruttan.  (1981).  Plant

Science (third edition).San Francisco CA:  McGraw-

Hill Book Co.  Pp. 208-212

Weaver, R. J.  (1976).  GRAPE GROWING.  New York, N.Y.:

J. Wiley & Sons Inc.  Pp. 142-145

University of California (pamphlet), "Questions

and Answers about Tensiometers," Leaflet # 2264,

March 1981.

Evaluation: Quiz by instructor

TOPIC PRESENTATION:  KNOWING WHEN TO IRRIGATE

A.  Irrigation

1.  Irrigation is capable of giving enormous benefits.

2.  Irrigation can also be a wasteful and harmful practice if not

applied correctly.

3.  The principles of irrigation are simple.

4.  It is difficult to give hard and fast rules however, because

individual crops and individual areas vary so greatly.

5.  It must be remembered that roots "don't grow to water", they grow

where water IS ALREADY present.

6.  To help decide when and how much water, the root zone of the

plant(s) must be defined.

7.  When irrigating, it is important to consider watering efficiently.

(Are you watering so far beyond the root zone you're going to waste

water?)

353.1

8.  If the goal is to have uniform plants in a crop; all those plants in

the crop must be watered the same way and amount.

a.  Cultivars are the one way to help insure uniformity (of course

this is assuming that water is applied uniformly).

b.  The ripening of the crop all at the same time improves ease of

management, increases efficiency of harvest and saves money.

B.  Knowing when to irrigate.  (Are irrigators "born" or can they be

"trained"  -  some feel a good irrigator, whether in the field or the

greenhouse is as much an artist, as an employee.)

1.  Determining when to irrigate and how much water to apply are the

main problems in irrigating.

2.  The time and frequency of irrigation is determined by:

a.  soil type,

b.  area climate,

c.  type of crop, and

d.  time of ripening and harvest (especially for fruit crops).

3.  The object of irrigation is to replace the water lost from the

field soil by transpiration plus evaporation minus the annual

rainfall (precipitation).

4.  Timing is critical.

a.  Snap beans, if under moisture stress during flowering and pod

formation, suffer a depressed yield because of ovule

abortion and flower abscission.

b.  Berries of grapes will not reach full size if the root zone is

at W.P. during the berry enlarging period.  (This will occur

even if adequate moisture is available later)

5.  There are several methods used to determine when to irrigate.

a.  Experience plays an important role here.

1)  Most crops will indicate that the individual plants are

under moisture stress by changing color - usually a duller,

more ashen green-grey.

2)  Leaves will curl.

3)  Shoot tips will become harder and darker sometimes with a

grey cast and sometimes lose turgor.

4)  The soil will feel and appear more crumbly and drier.

Included is a feel chart for determining moisture in

medium to fine textured soil:

353.2

============================================================================

DEGREE OF MOISTURE               FEEL                    PERCENT OF

FIELD CAPACITY

____________________________________________________________________________

Dry                       - Powder dry                           0

Low (critical)            - Crumbly, will not form a ball       Less than 25

Fair (usual time          - Forms a ball, but will crumble       25  -  50

to irrigate)                upon being tossed several times

Good                      - Forms a ball that will remain        50 - 75

intact after being tossed 5 times;

will stick slightly with pressure.

Excellent                 - Forms a durable ball and is          75 - 100

pliable; sticks readily; a sizable

chunk will stick to the thumb after

soil is squeezed firmly.

Too wet                   - With firm pressure, some water      In excess of

can be squeezed from the ball.      field

capacity

**SOURCE:  Strong.  (1956)  Sprinkler Irrigation Manual.  England: Wright

Rain & Ringwood.

============================================================================

6.  The use of "tensiometers" are proving especially useful in

permanent planting like orchards and vineyards.

a.  The tensiometer consists of a porous cup filled with

water that can be buried to a desired depth in the soil

in the vicinity of roots.

b.  The cup is connected by a water filled tube to a vacuum gage.

c.  As the soil dries, it sucks water out through the porous

wall of the cup, creating a partial vacuum inside the

tensiometer that can be read on the vacuum gauge.

d.  As soil moisture is replaced, water reenters the porous cup

relieving the vacuum tension.

7.  Many of our California crops require close to 3 feet (36 inches) of

water to bring the crop to proper harvest stage.

8.  Water not received by natural precipitation must be furnished

by irrigation.

353.3