- Importance of Leveling

  - The determination of elevations is called leveling

  - Measuring relative elevations changes is a comparatively simply process

  - Precise and accurate control of relative elevations are critical to most construction projects

- The Level

  - A level consist of a high-powered telescope

  - The level is attached to a spirit or bubble level that keeps the line of sight of the telescope horizontal

  - Level Instruments

  - Reading the Rod

- Definitions

  - Bench mark (BM) - relatively permanent 
point of known elevation

  - Backsight (BS) - a sight taken to the level rod held at a point of known elevation 
(either a BM or TP)

  - Height of instrument (HI) - the elevation of the line of sight of the telescope

  - Foresight (FS) - a sight taken on any point to determine its elevation

- Computation of Elevations

  - BS + Elevation  = HI

  - HI – FS= Elevation

  - Change in elevation- summation of the backsight and the foresight then subtract

  - The initial backsight (BS) is taken to a point of known elevation

  - The backsight reading is added to the elevation of the known point to compute the height of the instrument (HI)

  - The level may be moved to a temporary point called a turning point (TP)

  - The elevation of a point is the height of the instrument (HI) minus the foresight (FS)

- Common Mistakes

  - Misreading the rod - reading 3.54 instead of 3.45

  - Moving the turning point - use a well–defined TP

  - Field note mistakes - work within your group to check you records

  - Mistakes with extended rod - make sure the leveling rod is fully extended

  - Level rod not vertical

  - Settling of leveling rod

  - Leveling rod not fully extended or incorrect length

  - Level instrument not level

  - Instrument out of adjustment

  - Environment - wind and heat